- Joseph Maher Labib Estawro – 3699049

- Divyansh Sharma – 3758980

- Sanan Akther - 3704154




1. Architectural Overview

The provided architecture splits the implementation into a two-tier hybrid framework :

- Python Host Driver ( Topic_2_CTreconstruction.py): Manages file I/O operations (HDF5 loading and saving), configures physical CT acquisition geometries (calculating standard voxel sizes, source-to-object distances, source-to-detector distances, and projection angles), tracks processing metrics, and runs the iterative solver loop.

- Native C++ Accelerated Engine ( backend.cpp): Handles the high-computational processing via native C++ operators linked dynamically to the Python runtime environment via PyTorch's Just-In-Time (JIT) loadutility with high vectorization flags ( -O3) and OpenMP configurations ( -fopenmp).

## 2. Structural Fixes & Core Adaptations

### A. Resolution of the PyTorch Tensor Accessor Sub-Slice Crash

- The Problem: Modern versions of PyTorch explicitly forbid fetching raw memory accessors from temporary sub-slice structures within loop systems (eg, calling .accessor()inside an array projection grab projs[p]). Doing so triggers compilation errors due to safety provisions against pointer invalidation.

- The Change: The multi-dimensional slicing was replaced by creating a unified 3D array memory mapping ( projs.accessor<float,3>()) outside the thread execution scope. Inside the innermost loops, the data values ​​are parsed through explicit index calculations across the raw memory layer, resolving the build failure while maintaining maximum processing speeds.

## 3. High-Performance Execution Transformations

### A. Algorithmic Coordinate Flattening & Value Hoisting

- Before: The original forward projection engine performed raw math formulas (such as (px + svx / 2.0f) / svx * nvx - 0.5f) and Euclidean norms ( std::sqrt) repeatedly inside the deepest sample loop layer ($n\_samples$).

- After: Computational expressions were expanded and flattened into static constants ( scale_x, shift_x, scale_y, etc.) calculated once outside the loop. The std::sqrtcalculations were hoisted out of the sample loops, changing deep nested math updates into single-cycle fused multiply-add (FMA) instructions.

### B. Inversion of Nested Loops for Memory Cache Locality

- Before: The backprojection algorithm was organized natively as Voxel (X) -> Voxel (Y) -> Voxel (Z) -> Projection (P). Because individual projection images reside far apart in system memory, iterating over the projection index pinside the innermost layer forced the CPU to repeatedly load and clear its L1/L2 caches (known as cache thrashing).

- After: The entire loop structure was completely inverted. The projection loop iteration pwas elevated to the absolute outermost layer , while the spatial voxel structures were pushed inside. A massive projection array is now fetched into the high-speed CPU cache only once , feeding its energy sequence across all underlying voxel coordinates seamlessly without memory bandwidth delays.

### C. Fast division via reciprocal estimation

- Before: The engine recalculated geometric scaling factors on every voxel by dynamically dividing by the projection depth coordinate U( SDD * t_arr[p] / u_val). On modern CPUs, hardware division operations are remarkably resource-heavy compared to basic arithmetic.

- After: The system calculates the localized inverse of the distance array once ( 1.0f / U) using a dedicated floating-point reciprocal estimation. All subsequent divisions inside the nested spatial system are handled via low-overhead floating-point multiplications ( * u_inv), accelerating processing speeds.

### D. Private Thread Buffering for Non-Blocking Multi-Threading

- Before: Inverting the loop structure meant multiple concurrent OpenMP threads could attempt to modify the exact same voxel coordinates simultaneously, creating dangerous race conditions. Securing these memory addresses using standard hardware blocks ( #pragma omp atomic) would slow down execution by forcing threads to wait for one another.

- After: The implementation now dynamically reads the maximum hardware core capacity ( omp_get_max_threads()) and allocates a private 3D array buffer for each operational thread. Threads execute simultaneously inside independent memory sectors without blocking. A highly efficient memory reduction summarizes the sub-volumes into the master record at the very end.